| Control Systems Lab EE-303 Engr. M.UsmanSardar | Swedish College of Engineering and Technology, RYK Punjab, Pakistan |
| --- | --- |

Control Systems (EE-303)

Electrical Power Engineering

5th Semester

Batch 2011-2015

- Student Name: 	________________________

- Class Roll No: 	________________________

- Exams Roll No:	________________________

Verification of Instructed: ____________________

Grading: ____________________

Dated: ____________________

Electrical Engineering Department, SCET – RYK

Affiliated with TheIslamia University of Bahawalpur

LIST OF LAB EXPERIMENTS

Course: Control Systems (EE-303)

Swedish College of Engineering and Technology, Rahim Yar Khan

SERIAL # 				EXPERIMENT

- Introduction to MATLAB and software utility

- Creating the Transfer Functions using different commands

- Creating the transfer function of different Electrical, Mechanical and Rotational systems

- Creating the State Equations and Inter-conversion of the Transfer-function into State-Space Representation

- To Connect transfer function Blocks Using MATLAB

- To Observe The Response of First-ORDER Systems and  to observe The Response Of Second Order System Regarding Damping

- To Observe The Response Of Second Order System by changing the Poles

- Check the System Responses with Additional poles

- Check the System Responses with Additional Zeros

- Check the response of Zero pole Cancellation using residues

- Study of ON/OFF controller

- Study of open, close loop system and close loop system with disturbance

- Study of Proportional, Integrator and Derivative controllers individually

- Study of Proportional + Integrator (PI), and PD controllers

- Study of Proportional + Integrator + Derivative(PID) controller

- Study of Proportional + Integrator + Derivative(PID) controller in close loop

- Study of Proportional + Integrator + Derivative (PID) controller with 1st and 2nd order systems

Lab Instructor: Engr. Muhammad Usman Sardar

Computer Laboratory- Matlab 8ver.

Department of Electrical Power Engineering, SCET RYK

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 01)

# Introduction to MATLAB

PERFORMANCE OBJECTIVES:-

Upon successful completion of this experiment student will be able to:

- Become familiar with MATLAB environment

- Become familiar with Basic MATLAB commands

- Write short logical programs using MATLAB

EQUIPMENT REQUIRED:

- A computer running Matlab 7.0 or higher version

THEORETICAL BACKGROUND:-

- Matlab is an interactive system for doing numerical computations.

- A numerical analyst called Cleve Moler wrote the first version of Matlab in the 1970s. It has since evolved into a successful commercial software package.

- Matlab relieves you of a lot of the mundane tasks associated with solving problems numerically.

- This allows you to spend more time thinking, and encourages you to experiment.

- Matlab makes use of highly respected algorithms and hence you can be confident about your results.

- Powerful operations can be performed using just one or two commands.

- You can build up your own set of functions for a particular application.

- Excellent graphics facilities are available, and the pictures can be inserted into LATEX and Word documents.

MATLAB is a high level computer language used for the programming of complex engineering problems. MATLAB stands for MATrix LABoratory. In MATLAB we have many tool boxes each containing functions related to specific operations, just as we have libraries in JAVA or C language. Communications, Control System, Image processing and Image acquisition are some of the most commonly used tool boxes. MATLAB has extensive capabilities of two and three dimensional plotting. We will use these plotting facilities of MATLAB to aid in the physical interpretation of equations, particularly those related to fields and potentials.

In MATLAB, there are two modes of executing a program; the first one is command line execution and the second one is M-File or DOT M (.m) file execution. For command line execution commands are entered in the command window where (.m) file, complete program is first written in a file and saved with DOT M (.m) extension. The complete program is then RUN like any other programming language. We will use DOT M files throughout this lab.

BASIC TERMINOLOGY:

MATLAB has some basic terms and predefined variables you should get familiar with before going any further. One of these is the variable ans.  When you type most commands to MATLAB, they will return values.  You can assign these values to variables by typing in equations. For example, if you type

>>x = 5

MATLAB will print

x =

5

If you terminate a command with a semi-colon, MATLAB will suppress the printing of the variable name and value resulting from the calculation. For example, if you type

>>x = 5;

MATLAB will assign the value five to the variable x, but rather than tell you it did that, it will just return another << prompt. MATLAB works with two basic types of data objects: scalars and matrices.  MATLAB also has vectors.  Vectors are a special case of matrices which are only 1 row by any number of columns.  To assign x to be a matrix by explicitly entering the elements, you type the list of elements separated by blanks or commas surrounded by [ and], and use semi-colons to separate the rows. For example, typing

>>x = [2  4  6  8  ;  1  3  5  7]

Results in

x =             2        4        6        8

1        3        5        7

The MATLAB workspace is defined as the collection of all the variables you have defined during the current MATLAB session.

BASIC ARITHMETIC:

MATLAB uses a straightforward notation for basic arithmetic on scalars.  The symbol + is used to add scalars, so x = 1 + 5 will give x the value 6.  Similarly, MATLAB uses – for subtraction, * for multiplication, / for division, and ^ for exponentiation.  All of these work for two scalars. In addition, you can add, subtract, multiply or divide all the elements of a vector or matrix by a scalar.  For example, if x is a matrix or vector, then x+1 will add one to each element of x, and x/2 will divide each element of x by 2.  all of the basic operations ( =,   -,  *,   /, and  ^) are defined to work with complex scalars.

Another useful operator is the colon. You can use the colon to specify a range of numbers. Typing

>>x = 1 : 4

Will return

X=

1      2         3       4

MATLAB HELP:

MATLAB has a fairly good help facility.  The help function knows about all the commands listed in this manual.  Typing help function will tell you the syntax for the function, i.e. what arguments it expects.  It will also give you a short description of claims you are in error, try looking at the help for the functions you are using.

Check MATLAB help for PLOT,  FOR,  SUBPLOT,  ROOTS.

>>help plot

>>help for

>>help subplot

>>help roots

Try the following commands

>>doc plot

>>doc for

>>doc subplot

>>doc roots

roots

Polynomial roots

Syntax

r = roots(c)

Description

r = roots(c) returns a column vector whose elements are the roots of the polynomial c. Row vector c contains the coefficients of a polynomial, ordered in descending powers. If c has n+1 components, the polynomial it represents is C1Sn + C2Sn-1   + C3 Sn-2  + ……+CnS + Cn+1

Examples:-	The polynomial S3+S2+S-27 is represented in MATLAB as

p = [1 -6 -72 -27]

The roots of this polynomial are returned in a column vector by

>>r = roots(p)

r =

12.1229

-5.7345

-0.3884

Suppressing the output:-

One often does not want to see the result of intermediate calculations---terminate the assignment statement or expression with semi-colon

>> x= -13; y = 5*x,  z = x^2+y

y =

-65

z =

104

>>

The value of x is hidden. Note also we can place several statements on one line, separated by commas or semi-colons.

Products, Division & Powers of Vectors

We can perform this product in Matlab by

>> u = [ 10, -11, 12], v = [20; -21; -22]

>> prod = u*v 	% row times column vector

prod =

167

>>

Suppose we also define a row vector w and a column vector z by

>> w = [2, 1, 3], z = [7; 6; 5]

w =

2 1 3

z =

7

6

5

and we wish to form the scalar products of u with w and v with z.

>>u*w

??? Error using ==> *

Inner matrix dimensions must agree.

There is no mathematical definition for the division of one vector by another. However, in Matlab, the operator ./ is defined to give element by element division—it is therefore only defined for vectors of the same size and type.

>> a = 1:5, b = 6:10, a./b

a =

1 2 3 4 5

b =

6 7 8 9 10

ans =

0.1667 0.2857 0.3750 0.4444 0.5000

>> a./a

ans =

1 1 1 1 1

>> c = -2:2, a./c

c =

-2 -1 0 1 2

Warning: Divide by zero

ans =

-0.5000 -2.0000 Inf 4.0000 2.5000

User Defined Function

Now we will draw the user defined function. i.e if we are to draw the exponential function

x(t) =eat+jθt for a andθ inputs for t.

MATLABB CODE

a=input('enter the a = ');

b=input('enter the phi = ');

T_min=input(' Enter the starting time = ');

T_ss=input(' Enter the Step Size on time interval = ');

T_max=input(' Enter the ending time = ');

t=T_min:T_ss:T_max;

>>title('Graph of complex exponential)')

>>xlabel('x axis')

>>ylabel('y-axis')

x_t= exp(-a.*t + i*b.*t);

plot(t,x_t)

Figure 1.01 plot for a given values of

A and θ

SOLUTION OF DIFFERENT PROBLEMS

Q # 01 Plot the following functions, individually on a separate plane then combine on a single plane.

- u(t)=0 ;for t<0

u(t)=5 ;for t>0

Solution:

t=-5:.0001:0;

t_1=0:.0001:5;

u=0;

u_1=5;

plot(t,x,t_1,x_1 )

axis([-5 5 -1 6])

Figure: 1.02

Q # 02 plot following on a single – page separately using the command “subplot”

Solution:

In m-file

a=0:0.0001:20;

w=20.*exp(-2.*a);

b=0:.0001:8;

x=sin(pi.*b/4);

c=0:.0001:8;

y=cos(pi.*c/4);

subplot(221)

plot(a,w)

xlabel('time axis')

ylabel('w')

subplot(222)

plot(b,x)

xlabel('time axis')

ylabel('x')

subplot(223)

plot(c,y);

xlabel('time axis')

ylabel('y')

PLOT

Figure 1.02 subplot

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 02)

# Creating the Transfer Functions in MATLAB

PERFORMANCE OBJECTIVES:-

Upon successful completion of this experiment student will be able to:

- Become familiar with transfer function with different Functions

EQUIPMENT REQUIRED:

- A computer running Matlab 7.0 or higher version

THEORETICAL BACKGROUND:-

Creating the Polynomials expressions

- Creating the polynomials as a row vector containing the coefficients, thus s^3+3s^2+3s+1

can be represented by the vector of coefficients shown below with elements separated by space or comma

>>P1=[1 3 3 1] %store polynomial will be s^3+3s^2+3s+1

>>Roots(P1)

-1.00

-1.00

-1.00

- In factored form, we will use roots of polynomials

P2= ploy([-1 -1 -1])

ans =

1.0000    3.0000    3.0000    1.0000

% stored polynomial is s^3+3s^2+3s+1

% here P1 and P2 are identical

- Multiplication of polynomials using conv command

>> P3=conv(P1,P2)

P3 =

1     6    15    20    15     6     1

- Partial fraction expression

Partial fraction expression can be found for F(s)= b(s)a(s)  using [K p  k]= residue(b,a) command (k=residue ; p=roots of denominator ; k=direct quotient ), which is found by dividing polynomials prior to performing a oartial fraction expansion) . we expand

F(s) = 7s2+9s+12 S(S+7)(S2+10s+100) (=7s2+9s+12 s4+17s3+170s2+700 s+0 )

M.C :

Numf= [7 9 12] % define numerator of F(s)

Denf=conv(poly([0 -7]) , [1 10 100]  )

[K p k]= residue(Numf, Denf)

K =

0.2554 - 0.3382i

0.2554 + 0.3382i

-0.5280

0.0171

p =

-5.0000 + 8.6603i

-5.0000 - 8.6603i

-7.0000

0

k =

[]

- Vector Method, Polynomial form

A transfer function can be expressed as a numerator polynomial divided by denominator, that is

G(s) =C(s)R(s)

Numerator C(s) is represented by a row vector ‘num’, that contains the coefficients of c(s). Similarly the denominator R(s), is represented by a row vector ‘den’, that contains the coefficients of R(s). We form G(s) with the command

G=tf(num,den)	(G is called LTI object )

MATLAB CODE:

>>numg1 = 1; deng1 = [1 1]; numg2 = 1; deng2 = [1 2];

>>G1=tf(numg1,deng1)

>> G1

Transfer function:

1

-----

s + 1

>>G2 = tf(numg2,deng2);

>> G2

Transfer function:

1

-----

s + 2

- Vector Method, Factored form

We can create transfer function of if numerator and denominator are given in factored dorm. We do this row vector containing the roots of numerator and denominator. Thus

G(s) =  K C(s)R(s)

G=zpk(num,den,k)

MATLAB CODE

>>den=[-2 -4];

>>num=[-7 -8 -9];

>> k=20;

>> G_1=zpk(den,num,k)

Zero/pole/gain:

20 (s+2) (s+4)

-----------------

(s+7) (s+8) (s+9)

- Rational Expression in s method, Polynomial form

This method allows us to type transfer function as we normally would write it. The statement s=tf(‘s’)must precede the transfer function if we wish to create and LTI transfer function in polynomial form equivalent to using G=tf(num , den)

Let,

>> s=tf('s');

>> h=(s+1)/(s^2+7*s+12)

Transfer function:

s + 1

--------------

s^2 + 7 s + 12

- Rational Expression in s method, factored form

This method allows us to type transfer function as we normally would write it. The statement s=zpk(‘s’)must precede the transfer function if we wish to create and LTI transfer function in polynomial form equivalent to using G=zpk(num , den)

Let,

>> s=zpk('s');

>> h=(s+1)/(s^2+7*s+12)

Zero/pole/gain:

(s+1)

-----------

(s+4) (s+3)

Conversion Between polynomial form and factored form

Transfer function num and den can be converted between polynomial form containing coefficients and factored form containing the roots.

We will use command

tf2zp(num,den)

- write the polynomial in “zpk” and “tf”  form

5s2 + 10s +5

S2 + 6s + 9

MATLAB CODE

>>num= [5 10 5];

>>den= [1 6 9];

>> [z p k]= tf2zp(num,den)

z =

-1

-1

p =

-3.0000 + 0.0000i

-3.0000 - 0.0000i

k =

5

And if we convert back this zero/pole/gain to transfer function, then we will use the codes

>> [num,den]=zp2tf(z,p,k)

num =

5    10     5

den =

1.0000    6.0000    9.0000

This is the actual polynomial

5s2 + 10s +5

S2 + 6s + 9

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 03)

# Creating the transfer function of different systems

PERFORMANCE OBJECTIVES:-

Upon successful completion of this experiment student will be able to:

- discuss the different system and  its Laplace I.e., Electrical, Mechanical systems

- Creating the transfer function of different system

EQUIPMENT REQUIRED:

- A computer running Matlab 7.0 or higher version

THEORETICAL BACKGROUND:-

The transfer function is often described in two multiplied parts, the static part and the dynamic part. The static transfer function describes the input/output relationship when the input and output are not changing in time. It can be linear or non-linear, in equation or in tabulation form.

Static transfer function =System Input in steady state System output in steady state

The dynamic transfer function describes the input/output relationship when there is a time variation for the system variables (during the transient response).

Firstly we consider the Electrical System shown in figure; we will find out its transfer function

Fig: Given Electrical System

Here 						VL + VR + VC= e i

The transfer function model of circuit can be obtained by taking the Laplace transform of equation, assuming the zero initial condition. We get

Then the transfer function E o (s)E i (s) becomes as

MATLAB CODE:

>> R=10000; L=200e-3; C=200e-6;

>>den= [1]

>>num = [ L*C  R*C  1 ]

>>Tf1= tf(den,num)

Fig: plot of Step Response

Mechanical System:

Consider the mechanical system, find the transfer function X(s) / F(s) .

Laplace transform of the differential equation of motion derived using newton law of motion.

Solution of above equation yields the following transfer function of system

Matlab Code:

M= 2; fv= 1; K=4;

Num= [1];

Den=  [ M  fv   K ];

G= tf(Num ,Den )

Step(G,3)

Fig: plot of Step Response

Exercise question:

Consider the Cascaded system shown in Figure below. Assume the (e )i  is the input and  (e )o  is the output. The capacitances C1 and C 2 are not charged initially. Assume different values of components. Find the transfer function of system.  And observe the step output.

Figure: Electrical system

(Hint)

The output transfer function will be

Solution:

Problem 02: (mechanical System)

For the system, derive the transfer function for the given step input of torque T(t)

(Hint)

The transfer function for the system is

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 04)

# Creating the State Equations and Inter-conversion of the Transfer-function into State-Space Representation

PERFORMANCE OBJECTIVES:-

Upon successful completion of this experiment student will be able to:

- Creating the State-space Functions

- Inter-conversion between transfer function and state space

EQUIPMENT REQUIRED:

- A computer running Matlab 7.0 or higher version

THEORETICAL BACKGROUND:-

State-Space Representation of LTI Systems:

In control engineering, a state space representation is a mathematical model of a physical system as a set of input, output and state variables related by first-order differential equations. To abstract from the number of inputs, outputs and states, the variables are expressed as vectors, and the differential and algebraic equations are written in matrix form (the last one can be done when the dynamical system is linear and time invariant). The state space representation (also known as the "time-domain approach") provides a convenient and compact way to model and analyze systems with multiple inputs and outputs. With p inputs and q outputs, we would otherwise have to write down p×qLaplace transforms to encode all the information about a system. Unlike the frequency domain approach, the use of the state space representation is not limited to systems with linear components and zero initial conditions. "State space" refers to the space whose axes are the state variables. The state of the system can be represented as a vector within that space.

The State Equations

A standard form for the state equations is used throughout system dynamics. In the standard form the mathematical description of the system is expressed as a set of n coupled first-order ordinary differential equations, known as the state equations, in which the time derivative of each state variable is expressed in terms of the state variables x 1(t), . . . , x n(t) and the system inputs u1(t), . . . , u r(t). In the general case the form of the n state equations is:

wherex˙i= dxi/dtand each of the functions fi (x, u, t), (i = 1, . . . , n) may be a general nonlinear, time varying function of the state variables, the system inputs, and time. It is common to express the state equations in a vector form, in which the set of n state variables is written as a state vector x(t) = [x1(t), x2(t), . . . , xn(t)] T , and the set of r inputs is written as an input vector u(t) = [u1(t), u2(t), . . . , ur(t)]T. Eachstate variable is a time varying component of the column vector x(t).

This form of the state equations explicitly represents the basic elements contained in the definition of a state determined system. Given a set of initial conditions (the values of the xi at some time t0) and the inputs for t≥ t0, the state equations explicitly specify the Derivatives of all state variables. The value of each state variable at some time Δtlater may then be found by direct integration.

The system model in the form:

x˙ = Ax + Bu

y = Cx+ Du.

Representation in MATLAB

Given the Electrical network Shown in figure 4.01. Find its State Space representation ,Output is Across VC and create it in MATLAB.

Figure: 4.01

Hare are the two state variables i.e., VC and IL.

Ic =C dVcdt  …….(i)		 and

VL=LdILdt  …….(ii)

Using the Loop of Network and finally get the representations as

dILdt = V(s)L- VLL- R ILL …..(iii) And generates the following pair of state equations:

The required output equation is:

MATLAB code:

>> R=10000;L=200e-3;C=200e-6;

>> A=[0 1/C;-1/L -R/L];B=[0;1/L];C=[0 1];D=[0];

>> u=ss(A,B,C,D);

>>u

a =

x1       x2

x1        0     5000

x2       -5  -5e+004

b =

u1

x1   0

x2   5

c =

x1  x2

y1   0   1

d =

u1

y1   0

Continuous-time model.

>> [num den]=ss2tf(A,B,C,D);

>> G=tf(num,den)

Transfer function:

5 s + 2.443e-007

-----------------------

s^2 + 5e004 s + 2.5e004

when we put R=0 then Output will be Un=da

MATLAB CODE:-

>> R=10000;L=200e-3;C=200e-6;

>> R=00;L=200e-3;C=200e-6;

>> A=[0 1/C;-1/L -R/L];B=[0;1/L];C=[0 1];D=[0];

>> u=ss(A,B,C,D);

>>step(u)

>>step(u,0.1)

Figure 4.02

Exercise Problem # 01

Given the Electrical Network of figure, find the state space representation and if the Output is the current through the resister. Suppose your own values of components &then convert it into transfer-function.

Fig: Electrical Network for the representation of in State space:

Exercise Problem # 02

Given the figure, find the state space representation of translation mechanical system for output f(t).  Suppose your own values of components & then convert it into transfer-function.

Fig: the given system

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 05)

# To Connect Blocks Using MATLAB

PERFORMANCE OBJECTIVES:-

The MATLAB’s Control System toolbox provides a number of functions to help us build complex networks of models. We will able to perform the followings

- Series Connection

- Parallel Connection

- Feedback Connection

EQUIPMENT REQUIRED:

- A computer running Matlab 7.0 or higher version

THEORETICAL BACKGROUND:-

Series Connection

Consider the series network shown in Fig. 3-1.

We can connect block G1 and G2 in series using command G = series(G1,G2) A series connection is equivalent to the product of the blocks.

G = G1 * G2

The following MATLAB program connects these blocks in series.

numg1 = 1;

deng1 = [1 1];

numg2 = 1;

deng2 = [1 2];

G1 = tf(numg1,deng1);

G2 = tf(numg2,deng2);

G = series(G1,G2)

This returns;

Transfer function:

1

-------------

s^2 + 3 s + 2

Parallel Connection

Consider the parallel network shown in Fig. 3-2

Blocks G1 and G2 can be connected in parallel using command

G = parallel(G1,G2)

A parallel connection is equivalent to the sum of the blocks.

G = G1 + G2

Taking the same block values as in first program, parallel connection is obtained

numg1 = 1;

deng1 = [1 1];

numg2 = 1;

deng2 = [1 2];

G1 = tf(numg1,deng1);

G2 = tf(numg2,deng2);

G = parallel(G1,G2)

This returns;

Transfer function:

2 s + 3

-------------

s^2 + 3 s + 2

Feedback Connection

Consider the feedback diagram shown in Fig. 3-3.

We can connect blocks G and H in feedback configuration using Commandfeedback(G,H)

The negative feedback is default.

To connect in positive feedback configuration

feedback(G,H,+1)

Let for the above case

Negative feedback connection of these blocks in MATLAB can be obtained

numg = 1;

deng = [1 0];

numh = 1;

denh = [1 3];

G = tf(numg,deng);

H = tf(numh,denh);

cloop = feedback(G,H)

This returns;

Transfer function:

s + 3

-------------

s^2 + 3 s + 1

For positive feedback replace final line

of code by following code

cloop = feedback(G,H,+1)

We get;

Transfer function:

s + 3

-------------

s^2 + 3 s – 1

LAB Work

Q#01 Find the equivalent connection of following networks using MATLAB.

SOLUTION:	MATLAB code

>> a=tf([2 3],[1 2 3]);

>> b=tf([1],[1 0 0]);

>> c=series(a,b)

Transfer function:

2 s + 3

-------------------

s^4 + 2 s^3 + 3 s^2

Q#02

SOLUTION:	MATLAB code

>> a=tf([40 60 0 0],[1 0 1]);

>> b=zpk([-3],[0 -1 -2],1);

>> c=series(a,b)

Zero/pole/gain:

40 s^2 (s+3) (s+1.5)

------------------------

s (s+1) (s+2) (s^2  + 1)

Q#03

SOLUTION:

MATLAB code

>> a=zpk([],[-1 -2 -2],2);

>> b=zpk([],[0 -4 -8],32);

>> c=parallel(a,b)

Zero/pole/gain:

34 (s+0.5637) (s^2  + 4.848s + 6.679)

-------------------------------------

s (s+1) (s+2)^2 (s+4) (s+8)

Q#04

MATLAB code

>> a=zpk([-4 -20],[0 0 0 -100 -500],10);

>> b=zpk([],[0 -10 -20],1);

>> c=feedback(a,b)

Zero/pole/gain:

Zero/pole/gain:

10 s (s+20)^2 (s+10) (s+4)

-----------------------------------------------------------------------------------

(s+500) (s+100) (s+20) (s+10) (s^2 + 0.1338s + 0.008903) (s^2 - 0.1338s + 0.008986)

>>

Q#05

SOLUTION:

Hare we assume the negative feedback

MATLAB code

>> a=tf([7 21],[1 2 3 1]);

>> b=tf(1,1)

>> c=feedback(a,b)

Transfer function:

7 s + 21

-----------------------

s^3 + 2 s^2 + 10 s + 22

It’s verified.

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 06)

# To Observe The Response Of First-ORDER Systems

PERFORMANCE OBJECTIVES:-

Upon successful completion of this experiment student will be able to:

- Understand the 1st order systems without Zero

- Apply the different type of responses

EQUIPMENT REQUIRED:

- A computer running Matlab 7.0 or higher version

THEORETICAL BACKGROUND:-

A first Order systems without zero can be describe by the Transfer function, If the input is a uit step, where R(s) = 1S , the Laplace transform of the step response is C(s), where

Taking the C(s) = R(s) G(s) = as(s+a)

Hare “a” is Exponential Frequency. And 1a is time constant.

Taking the Inverse Transform, the the Step response is given by

C (t)= C f (t) + C  n (t)= 1-e-at

There are different types of response we can apply

- Steo response

- Iimpulse

- Linea simulation

- Bode

- Bode magnitude etc

To check the step response we have to do as follows

MATLAB CODE

numg1=[10];

deng1=[1 10];

x1=tf(numg1,deng1);

numg2=[5];

deng2=[1 5];

x2=tf(numg2,deng2);

numg3=[1];

deng3=[1 1];

x3=tf(numg3,deng3);

step(x1,x2,x3)

>>impulse(x1,x2,x3)

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 07)

# To Observe The Response Of Second Order System Regarding Damping

PERFORMANCE OBJECTIVES:-

Upon successful completion of this experiment student will be able to:

- Apply the different type of responses

- Calculate the poles and zeros of Transfer function

EQUIPMENT REQUIRED:

- A computer running Matlab 7.0 or higher version

THEORETICAL BACKGROUND:-

There is different type of response in 2nd order system

General 2nd order system is as follows

C(S)R(S)=    bs2+ a s+b  = ωn2s2+ 2ξ ω s+ ωn2

Response are

(1) Over Damped for ξ >1		(2) Under Damped for 0<ξ<1

(3) Undamped for ξ=0		(4) critically damped for ξ=1

We will generate “Continuous second-order systems “by using values of ‘damping ratio and natural frequency

Command

>>ord2

[A,B,C,D] = ord2(wn,z) generates the state-space description (A,B,C,D) of the second-order system

given the natural frequency wn= ωnand damping factor z  (ξ). Use ss to turn this description into a state-space object. [num,den] = ord2(wn,z) returns the numerator and denominator of the second-order transfer function. Use tf to form the corresponding transfer function object.

To generate an LTI model of the second-order transfer function with damping factor z=0.4 and natural frequency w=2.4 rad/sec  ,

[num,den] = ord2(2.4,0.4)

num =

1

den =

1.0000    1.9200    5.7600

sys = tf(num,den)

Transfer function:

1

-------------------

s^2 + 1.92 s + 5.76

To calculate to ZERO and POLE of this transfer Function

>>zero(sys)

ans =

Empty matrix: 0-by-1

>>pole(sys)

ans =

-0.9600 + 2.1996i

-0.9600 - 2.1996i

Program:

Over-damped:

A=tf([9],[1 9 9]);

subplot(421);

pzmap(A);

title('poles zeros map of over damped 2nd order system')

subplot(422);

step(A);

title('over damped response of 2nd order system')

Under-damped:

B=tf([9],[1 2 9])

Subplot(423)

pzmap(B);

title('poles zeros map of under-damped 2nd order system')

subplot(424);

step(B);

title('under-damped response of 2nd order system')

Un-damped:

C=tf([9],[1 0 9]);

subplot(425);

title('poles zeros map of un-damped 2nd order system')

pzmap(C);

subplot(426);

step(C);

title('un-damped response of 2nd order system')

Critically-damped:

D=tf([9],[1 6 9]);

subplot(427);

title('poles zeros map of critically-damped 2nd order system')

pzmap(D);

subplot(428);

step(D);

title('critically-damped response of 2nd order system')

Result:

Figure 5.1 subplot

Control Systems 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 08)

# To Observe The Response Of Second Order System by changing the Poles

We will check the Step Response of 2nd order under-damped systems as pole moves

- With  constant real part

- With constant imaginary part

- With constant damping ratio

EQUIPMENT REQUIRED:

- A computer running Matlab 7.0 or higher version

THEORETICAL BACKGROUND:-

- With  constant real part

In this case we will have same envelope and settling time

a1=zpk([],[-1+1j -1-1j],9)

a2=zpk([],[-1+2j -1-2j],9)

a3=zpk([],[-1+3j -1-3j],9)

subplot(221);

pzmap(a1)

subplot(222)

pzmap(a2)

subplot(223);

pzmap(a3)

>>ltiview

Figure: 7.01

(2)With constant imaginary part

In this case we will have same peak time

a4=zpk([],[-1+2j -1-2j],9)

a5=zpk([],[-2+2j -2-2j],9)

a6=zpk([],[-3+2j -3-2j],9)

ltiview

(3) With constant damping ratio

a1=zpk([],[-1+j  -1-j],2);

a2=zpk([],[-2+2j  -2-2j],8);

a3=zpk([],[-3+3j  -3-3j],18);

subplot(221)

pzmap(a1)

subplot(222)

pzmap(a2)

subplot(223)

pzmap(a3)

>>ltiview

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 09)

# Check the System Responses with Additional poles

We will check the Step Response of Higher order under-damped systems with additional poles

EQUIPMENT REQUIRED:

- A computer running Matlab 7.0 or higher version

THEORETICAL BACKGROUND:-

We can not calculate the value of peak time, rise time, settling time, percentage over Shoots. However under certain conditions a system with more than 2 poles or with zeros can be approximated as a second order system that has just 2 complex dominant poles. Once we justify this approximation the formals for peak time, rise time, settling time, percentage over Shoots can b applied to these higher order systems by using the location of the dominating poles.

Additional poles

Hare T 2 =a2 can be approximated as a 2nd order system,

MATLAB CODE

num1=[24.542]

den1=[1 4   24.542]

a1=tf(num1,den1)

num2=[245.42]

den2=[1 14  64.642 245.42]

a2=tf(num2,den2)

num3=[73.626]

den3=[1 7  36.542 73.626]

a3=tf(num3,den3)

subplot(221)

pzmap(a1)

subplot(222)

pzmap(a2)

subplot(223)

pzmap(a3)

ltiview

Figure: 8.01

Figure:8.01

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 10)

# Check the System Responses with Additional Zeros and Zero pole Cancellation using residues

We will check the Step Response of Higher order under-damped systems with additional zeeros

EQUIPMENT REQUIRED:

- A computer running Matlab 7.0 or higher version

THEORETICAL BACKGROUND:-

We can not calculate the value of peak time, rise time, settling time, percentage over Shoots. However under certain conditions a system with more than 2 poles or with zeros can be approximated as a second order system that has just 2 complex dominant poles. Once we justify this approximation the formals for peak time, rise time, settling time, percentage over Shoots can b applied to these higher order systems by using the location of the dominating poles.

With  zero, effect on the system

MATLAB CODE

a1=zpk([],[-10 -20],185.71 )

a2=zpk([-7],[-6.5 -10 -20],185.71 )

a3=zpk([-7],[-6.9 -10 -20],185.71 )

% hare a1 and a3 will be similar and a second order approximation is valid

ltiview

Fig: 9.01

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 11)

# Study of ON/OFF controller

Objective:

Study of ON/OFF controller

Procedure:

Procedure:

Connections for ON/OFF controller are as shown in the Fig. 4

1. After making, all required connection on board which on the power supply andstart the experiment.

2. Set 1V at t.p. 1 and apply it to SetPoint of ON/OFF controller.

3. From disturbance block, apply to variable input of ON/OFF controller and varythe knob of disturbance block as required.

4. Now measure the voltage at variable i/p of ON/OFF controller with digitalmultimeter given on board.

5. When knob of ON/OFF is at minimum position, you will find that LED willglow when voltage of variable goes slightly above the set point voltage, whichmeans differential gap, is less when knob is at minimum position.

6. When you will move knob of ON/OFF controller at maximum position you willfind the differential gap for ON and OFF is increased.

Observation and Calculation:

| Sr. No. | Set Point Voltage (SP) (V) | ON/OFF Knob level (%) | Disturbance voltage | LED ON Level | LED OFF Level |
| --- | --- | --- | --- | --- | --- |

Conclusion:

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 12)

# Study of open loop system, closed loop system and close loop system with disturbance

Objective:

- Study of open loop system:

Procedure :

Connections for open loop system are as shown in the Fig. 5

1. Here set voltage is given to error detector(ground the PV terminal) and the errorsignal given to P block with Kp at minimum position, output of P block given toadder with rest terminals grounded and measure the output (t.p.10) on digitalvoltmeter given on board.

2. Ground PV and inputs of summing block which are not in use.

3. After making, all required connection on board which on the power supply andstart the experiment.

4. By controlling the knob of set point, we can set + 10V at test point, t.p.l and canfind out relationship between output and input variables.

5. After finding the relationship between i/p & o/p amplitude of system measure theoverall gain of the system and gain of proportional block Kp.

6. Set t.p.l in such a manner so that the reading of the o/p is comfortable value, forinstance IV with best possible accuracy.

7. Now note the value of t.p.l to which output at t.p.10 is IV, repeat the measurementvarying I/p so that output is 2V.

8. Again, determine I/p voltage required for this second o/p.

9. The chain amplification is defined as the ratio of the o/p difference to I/pdifference.

10. Collect result for more values (+10 is saturation limit) in tabular form and find outthe relationship between I/P& O/P.

Conclusion: The o/p variable can be manually controlled by reading the instrumentconnected to the o/p and varying the amplitude of I/p until the o/p variable reaches therequired value. With the above concept of open loop system can be get cleared.

Observation and Calculation:

| Sr. No. | Set Point Voltage (SP) Input Voltage (V) | Voltage at TP4 V4 (V) | Voltage at TP5 V5 (V) | A=KP= V4V5 | Output Voltage Voltage at TP10 | Over all gain O/pI/p |
| --- | --- | --- | --- | --- | --- | --- |

Conclusion:

2. Study of close loop system

Procedure :

Connections for close loop system are as shown in the Fig. 6.

1. Here o/p t.p.10 is feedback to the error detector with feedback block so that ifoutput variable varies about the required signal, due to any cause, this variationis reported to I/p, through the feedback path. This gives more stable o/p thenopen loop system.

2. Ground inputs of summing block which are not in use.

3. After making, all required connection on board which on the power supply andstart the experiment.

4. Adjust the reference signal so that an o/p signal is obtained far from thesaturation limits.

5. To measure the closed loop gain of the system for different values of thefeedback rate. Here feedback rate can be set by the different position of theknob given with the feedback block.

6. To set the feedback rate first apply 2V at the i/p of feedback block and measurethe o/p of that block, set position of knob such that o/p should give IV, thatmeans feedback rate is 0.5. Repeat the observation for different feedback rate like 0.25, 0.5, 0.75 and 1.

7. Set IV at set point (t.p.l) and observe the o/p at t.p.10 with feedback rate of0.25. Then note the changes for different feedback rate.

8. Calculate the value of the closed loop gain through the formula:

9. G = A/(1+BA) Where: A - gain KP, B - V2/V1, VI is I/p voltage of feedbackblock & V2 is o/p voltage of feedback block

10. With this, compare the calculated values with the measured values.

Fig. 6

Observation and Calculation:

| Sr. No. | Set Point Voltage (SP) Input Voltage (I/p) (V) | Voltage at TP4 V4 (V) | Voltage at TP5 V5 (V) | A=KP= V4V5 | V2 | V1 | B= V2V1 (Feed- back gain) | Gain G | Output Voltage Voltage at TP10 (O/P) | Over all gain O/pI/p |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Conclusion:

- Study of close loop system with disturbance

Procedure:

Connections for close loop system with disturbance are as shown in the Fig. 7.

1. Here we are adding disturbances (E) in close loop system to find out the effectof it.

2. Ground inputs of summing block which are not in use.

3. After making, all required connection on board which on the power supply andstart the experiment.

4. As per the theory, the disturbances is present on the controlled variable with anamplitude reduce by the feedback factor, this can be proved with fewmathematical calculations, therefore:

5. C = [SetPoint x A/ (1+B A)] + E/ (1+B A)

6. Now measure o/p values for different position of knob connected in thefeedback path.

7. Compare the o/p values with the directly measured values.

Fig. 7

Observation and Calculation:

| Sr. No. | Set Point Voltage (SP) Input Voltage (I/p) (V) | Voltage at TP4 V4 (V) | Voltage at TP5 V5 (V) | A=KP= V4V5 | V2 | V1 | B= V2V1 (Feed- back gain) | E | Output Voltage Voltage at TP10 (O/P) | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Conclusion:

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 13)

# Study of Proportional controller, Integrator Controller and derivative controller

Objective:

- Study of Proportional controller

Procedure:

Connections for Proportional controller are as shown in the Fig.9.

1. After making, all required connection on board which on the power supply and start the experiment.

2. Ground PV and inputs of summing block which are not in use.

3. Set +0.5V at test point t.p. 1.

4. Apply set point to proportional input.

5. Check the output of proportional block with digital voltmeter given on board.

6. Vary slowly the Kp value, observe the change in the output, and find out the proportional band that is PB = 100/Kp.

7. In the same manner, we can check by applying square and triangular wave and compare the output with table no.l shown above.

Fig. 9

| Sr. No. | Set Point Voltage (SP) Input Voltage (V) | Voltage at TP4 V4 (V) | Voltage at TP5 V5 (V) | A=KP= V4V5 | Output Voltage Voltage at TP10 | PB= 100 KP | Over all gain O/pI/p |
| --- | --- | --- | --- | --- | --- | --- | --- |

Conclusion:

- Study of Integrator controller

Procedure:

Connections for Integrator controller are as shown in the Fig. 10.

1. After making, all required connection on board which on the power supply andstart the experiment.

2. Ground PV and inputs of summing block which are not in use.

3. Apply square wave to the input of integrator.

4. Check the output of integrator block with CRO and compare with the table no. 1shown above.

5. Vary slowly the KI value and observe the changes in the output.

6. In the same manner can check by applying triangular wave.

Fig. 10

Observations and Calculations:

Input Wave-form and corresponding output-wave form observed:

- Study of Derivative controller

Procedure:

Connections for Derivative controller are as shown in the Fig. 11.

1. After making, all required connection on board which on the power supply andstart the experiment.

2. Ground PV and inputs of summing block which are not in use.

3. Apply square wave to the input of Derivative.

4. Check the output of Derivative block with CRO and compare with the tableno.l shown above.

5. Vary slowly the KD value and observe the changes in the output.

6. In the same manner, we can check by applying triangular wave

Fig. 11

Observations and Calculations:

Input Wave-form and corresponding output-wave form observed:

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 14)

# Study of Proportional + Integrator (PI) controller and PD Controller

Objective:

- Study of Proportional + Integrator (PI)

Procedure:

Connections for Proportional + Integrator (PI) controller are as shown in the Fig. 12.

1. After making, all required connection on board which on the power supply andstart the experiment.

2. Ground PV and inputs of summing block which are not in use.

3. Apply square wave to the set point (SP).

4. Check the output t.p.10 of summing block on CRO that will look like as shownin Fig. 13.

5. Vary slowly the KP & KI value and observe the changes in the output.

6. In the same manner, we can check by applying triangular wave.

Fig.12

Fig.13

Observations and Calculations:

Input Wave-form and corresponding output-wave form observed:

- Study of Proportional + Derivative (PD)

Procedure:

Connections for Proportional + Derivative (PD) controller are as shown in the Fig.14.

1. After making, all required connection on board which on the power supply andstart the experiment.

2. Ground PV and inputs of summing block which are not in use.

3. Apply square wave to the set point (SP).

4. Check the output t.p.10 of summing block on CRO that will look like as shownin Fig. 15.

5. Vary slowly the KP & KD value and observe the changes in the output andcompare with table no.l showing the.

6. In the same manner, we can check by applying triangular wave.

Fig.14

Fig.15

Observations and Calculations:

Input Wave-form and corresponding output-wave form observed:

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 15)

# Study of Proportional + Integrator + Derivative (PID)

Objective:

PID controllers

Procedure:

Connections for Proportional + Integrator + Derivative (PID) controller are as shown in the Fig. 16.

1. After making, all required connection on board which on the power supply andstart the experiment.

2. Ground PV and inputs of summing block which are not in use.

3. Apply square wave to the set point (SP).

4. Check the output t.p.10 of summing block on CRO that will look like as shownin Fig. 17.

5. Vary slowly the KP, KI &Kd value and observe the changes in the output.

6. In the same manner, we can check by applying triangular wave.

Fig.16

Fig.17

Observations and Calculations:

Input Wave-form and corresponding output-wave form observed:

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 16)

# Study of Proportional + Integrator + Derivative (PID) in close loop

Objective:

PID in close loop

Procedure:

Connections for Proportional + Integrator + Derivative (PID) controller are as shownin the Fig. 18.

1. After making, all required connection on board which on the power supply andstart the experiment.

2. Ground inputs of summing block which are not in use.

3. Apply square wave to the set point (SP).

4. Check output of summing block on CRO, you will find that after adjusting Kp,Ki &Kd output signal will be same as input applied at SP of error detector .

5. In the same manner, we can check by applying triangular wave.

Fig.18

Observations and Calculations:

Input Wave-form and corresponding output-wave form observed:

Control Systems (EE-303) 5th Semester (3rd Year)

(LAB EXPERIMENT NO. 17)

# Study of Proportional + Integrator + Derivative (PID) with first order system and second Order system

Objective:

PID in close loop

- Study of PID with first order

Procedure:

First Apply IV to the first order system block and observe the transient response onDigital storage oscilloscope, you will observe the response as shown in Fig.20.

1. Then do the Connections for Proportional + Integrator + Derivative (PID) withfirst order system are as shown in the Fig. 19.

2. After making, all required connection on board which on the power supply andstart the experiment.

3. Ground inputs of summing block which are not in use.

- Check the transient response of t.p. 11 on Digital storage oscilloscope; you will observe the response as shown in Fig.21. You can easily observe the effect ofPID by adjusting Kp, Ki &Kd knobs.

Fig 20

Observations and Calculations:

Input Wave-form and corresponding output-wave form observed:

- Study of PID with second order system

Procedure:

First Apply 2V to the first order system block and observe the transient response on Digital storage oscilloscope, you will observe the response as shown in Fig.23.

1. Then do the Connections for Proportional + Integrator + Derivative (PID) with second order system are as shown in the Fig.22.

2. After making, all required connection on board which on the power supply andstart the experiment.

3. Ground inputs of summing block which are not in use.

4. Check the transient response of t.p.12 on Digital storage oscilloscope; you will observe the response as shown in Fig.24. You can easily observe the effect of PIDby adjusting Kp, Ki &Kd knobs.

Observations and Calculations:

Input Wave-form and corresponding output-wave form observed: